# Job Hazard Analysis

Facilities & Property Management Demolition Activities

* Communicate & Review:

Discuss the JHA with the crew before work starts and note any site-specific information.

|  | Task/Step | Hazards | Controls |
| --- | --- | --- | --- |
|  | 1. Ensure that any building materials that will be impacted during demo have been assessed for asbestos | - Asbestos exposure | - Consult asbestos survey results posted onsite. - Stop work if building materials to be impacted have not yet been assessed. - Reach out to the supervisor, project manager, and EHS. - Stop work if any new building materials are identified that may be asbestos-containing. |
|  | 2. For buildings constructed prior to 1978, identify any potential lead-containing materials or coatings that will be impacted | - Lead exposure | - Consult with EHS regarding lead survey results posted onsite and past exposure monitoring data. - In the absence of material sample results and exposure monitoring, presume painted materials to be lead-containing and follow lead safe work practices. Refer to PSU Lead Management Plan, Appendices B and C. - Reach out to the supervisor, project manager, and EHS if you have any questions. - Work in a childcare facility or residential building requires a certified Lead Renovation Repair & Painting (RRP) individual to complete the work following Lead RRP protocols. Refer to the PSU Lead Management Plan. - Stop work if any new building materials are identified that may be lead-containing. |
|  | 3. Area prep: Isolate utilities (power, water) | - Electrical hazards - Water damage | - Lockout tagout by electricians. - Water isolated/shut off by plumbers. - Refer to Don’t Hit It Form (To be finalized). |

|  | Task/Step | Hazards | Controls |
| --- | --- | --- | --- |
|  | 4. Area prep: Isolate work area and restrict access |  | - Post Construction Access Only/PPE Required signage. - Use tape/cones to block off the area. |
|  | 5. Area prep: Set up dust control methods | - Dust (silica) | - Follow Silica Exposure Control Plan for this task. - Set up zipwalls to contain dust. - Cover flooring with plastic; two layers so debris can be wrapped burrito style. - Cover furniture with plastic. - For dust control methods for presumed lead-containing materials, refer to PSU Lead Management Plan, Appendices B and C. |
|  | 6. Cutting and removing drywall; prying drywall loose | - Electrical hazard/plumbing lines - Tool hazards - Overexertion/back strain - Falls - Dust (silica, mold) - Eye/skin irritation - Cuts/lacerations from utility knife - Cuts from metal drywall corners | - Verify absence of electrical and plumbing behind wall before using any saws. Contact FPM electricians, FPM plumbers, your supervisor, and the project manager if you have any questions. - Use proper techniques and settings for drills, sawzalls, etc. Ensure proper tool maintenance. - Inspect cords and extension cords. - Use drywall lifts for large sheets, get help, lift with legs, keep back straight. - Cut seams before prying; score 4’ sections or smaller. - Use appropriate access equipment (ladders, baker scaffolds, lifts). Refer to JHAs for ladders, Baker scaffolds and lifts. - Follow Silica Exposure Control Plan for this task. - Wear N95 respirator and safety glasses, or wear half mask respirator if properly fit tested and trained. - Use dust-minimizing tools and wet methods, control dust at source. - Wear cut resistant gloves when handling sharp-edged sheets, metal drywall corners and utility knives. |
|  | 7. Debris removal | - Dust (silica, mold) - Overexertion/back strain - Eye/skin irritation - Cuts | - Follow Silica Exposure Control Plan for this task. - Wear N95 respirator and safety glasses. - Use dust-minimizing tools and wet methods, control dust at the source; have good ventilation. - Wrap up small debris ‘burrito style’ in a second sheet of plastic. |

|  | Task/Step | Hazards | Controls |
| --- | --- | --- | --- |
|  |  |  | - Use drywall lifts for large sheets, get help, lift with legs, keep back straight. - Cut and remove smaller pieces. - Use a dump buggy for debris; cover with plastic during transport. - Wear cut-resistant gloves when handling sharp-edged sheets and metal drywall corners. - Maintain clear pathways. |
|  | 8. Cleanup | - Slips/trips (debris) - Punctures (nails, screws) | - Clean work area. - HEPA vacuum debris. - Maintain clear aisles/walkways. |
|  | 9. Site-specific information | ○ | ➢ |
|  | Required Training - Asbestos Awareness - Lead Awareness - Silica Awareness - Ladder and Baker Scaffold - Scissor Lift, when applicable - Hazard Communication - Respirator, when required | Required Equipment - Locate nearest eyewash - First aid kit onsite - AED onsite for electrical safe off | Required PPE (Personal Protective Equipment) - Safety glasses - Work gloves, task specific - Safety shoes - Long pants - Safety helmet - Hearing protection, when needed - N95 respirator, voluntary use - Half mask respirator, when required |
| Contributors: Created: | Karen Barnack, Environmental Health & Safety; Gail Hamilton, FPM In-House Construction 3/26/2026 |  |  |