|  | ### Task/Step | ### Hazards | ### Controls |
| --- | --- | --- | --- |
|  | #### Ensure that any building materials that will be impacted during drywall installation have been assessed for asbestos | - Asbestos exposure | - Consult asbestos survey results posted onsite. - Stop work if building materials to be impacted have not yet been assessed. - Ensure there is an 8” minimum clearance from asbestos-containing fireproofing. - Reach out to the supervisor, project manager, and EHS. - Stop work if any new building materials are identified that may be asbestos-containing or if there is insufficient clearance from asbestos-containing fireproofing. |
|  | #### For buildings constructed prior to 1978, identify any potential lead-containing materials or coatings that will be impacted. | - Lead exposure | - Consult with EHS regarding lead survey results posted onsite and past exposure monitoring data. - In the absence of material sample results and exposure monitoring, presume painted materials to be lead-containing and follow lead safe work practices. Refer to PSU Lead Management Plan, Appendices B and C. - Reach out to the supervisor, project manager and EHS if you have any questions. - Work in a childcare facility or residential building requires a certified Lead Renovation Repair & Painting (RRP) individual to complete the work following Lead RRP protocols. Refer to PSU Lead Management Plan. - Stop work if any new building materials are identified that may be lead-containing. |
|  | #### Unloading and moving materials | - Overexertion/back strain - Foot injury from dropped materials, material cart | - Use drywall lifts for large sheets, get help, lift with legs, keep back straight. - Use a material cart. - Wear safety shoes. - Use a high visibility vest when unloading materials from a street vehicle. |
|  | #### Measure and cut drywall | - Cuts from knives - Eye irritation - Tool hazards - Cuts from sharp drywall edges | - Wear cut resistant gloves and safety glasses. - Use proper techniques and settings for saws. - Use safe cutting/scoring tools. - Ensure utility knives are fitted with sharp blades to eliminate excessive cutting force. - Use proper hand placement and be aware of hand placement throughout the task. - Position drywall on a stable surface to maintain control during cutting. |
|  | #### Drywall hanging; lifting and screwing | - Falls - Overexertion/back strain - Foot injury from dropped materials - Cuts from sharp drywall edges | - Use drywall lifts for large sheets, get help, lift with legs, keep back straight. - Use appropriate access equipment (ladders, scaffolds, lifts) for heights. Refer to JHAs for ladders, Baker scaffolds and lifts. - Use proper techniques and settings for drills, etc. - Wear gloves. - Wear safety shoes. |
|  | #### Finish work; applying joint compound and sanding | - Dust (silica) - Eye irritant; dust | - Complete Silica Exposure Control Plan template for this task. - Wear N95 respirators, voluntary. - Wear safety glasses. - Use dust-minimizing tools; control dust at the source. - Use a sanding tool equipped with HEPA vacuum. |
|  | #### Cleanup | - Slips/trips (debris) - Punctures (nails, screws) - Electrical hazards | - Clean work area. - Use a dump buggy for debris. - HEPA vacuum dust. - Inspect cords and extension cords. - Maintain clear aisles/walkways. |
|  | #### Site-specific information |  |  |
|  | ### Required Training - Asbestos Awareness - Lead Awareness - Silica Awareness - Ladder and Baker Scaffold - Scissor Lift, when applicable - Hazard Communication | ### Required Equipment - Locate nearest eyewash - First aid kit onsite | Required PPE (Personal Protective Equipment) - Safety glasses - Work gloves, task specific - Safety shoes - Long pants - Safety helmet - Hearing protection, when needed - N95 respirator, voluntary use |
| ### Contributors: ### Created: | Karen Barnack, Environmental Health & Safety; Gail Hamilton, FPM In-House Construction 3/26/2026 |  |  |